Feed consumption = herd size * feed rate

Milk production rate = herd size* milk per cow + feed rate

Cooling_Water_Rate=herd size×cooling_water_per_cow

Water Use/Gain Table

| Activities | Water Use Equations | References |
| --- | --- | --- |
| Drinking | FWI = −60.2 + (1.43 × Milk) + (0.064 × NaK) + (0.83 × DM%) + (0.54 × TMP) + (0.08 × DIM) | https://www.sciencedirect.com/science/article/pii/S002203021630371X |
| Cooling | The cooling rate is simply given by water/cow. The water depends on THI (temperature-humidity index). If we assume that THI < 68 (no heat stress), water consumption = 0. However, if we assume THI > 80, the literature says 15-20 gallons/cow/day. | https://www.canr.msu.edu/news/water_use_for_heat_abatement |
| Surface Water | Precipitation = surface area * avg rainfall | The references here vary by farm. |
| Water Usage Rate | Water intake, lbs/day = 35.25 + 1.58 × Dry matter intake (lbs/day) + 0.90 × Milk yield (lbs/day) + 0.11 × Sodium intake (grams/day) 2.65 × Weekly mean minimum temperature (°F/1.8 – 17.778) | https://pubs.nmsu.edu/_d/D107/index.html |

Energy Use Table

| Activities | Energy Use Equations | References |
| --- | --- | --- |
| Milk Cooling | Milk production rate * cooling amount | https://attra.ncat.org/publication/dairy-farm-energy-efficiency/ This source assumes a cooling rate of .03 kWh per kg of milk |
| Lighting + Electricity | Herd Size * 1.5 | https://energytools.sc.egov.usda.gov/ assumes 1.5 kWh/cow/day for LED lighting |
| Biogas production rate | Manure storage * .05 | https://extension.psu.edu/biogas-from-manure Assumes ~0.05 $m^3$ biogas per kg of manure |
| Solar generation rate | Solar panel area * n * solar irradiance | https://pvwatts.nlr.gov/ n is efficiency, typically 0.15–0.20 for standard PV |
| Sodium Intake | .0018 * Dry Matter Intake | https://pubs.nmsu.edu/_d/D107/index.html |

Lactating cows only, treat as one system only, not aggregate

Feed Use Table

| Activities | Feed Use Equations | References |
| --- | --- | --- |
| Feed consumption rate | Herd_size * (bodyweight * intake factor) | Intake factor is based on body weight: Nutrient Requirements of Dairy Cattle: Eighth Revised Edition (2021) |
| Forage inflow | Feed consumption rate * .55 | 55% forage |
| Grain inflow | Feed consumption rate * .45 | 45% grain |
| Dry Matter Intake | (Forage Inflow* %DMforage + GrainInflow*%DMGrain)/ (#Herd_Size) | https://utia.tennessee.edu/publications/wp-content/uploads/sites/269/2023/10/pb1598.pdf Assumes that variables are tracked for each cow, if not, divide equation by #of cows |

- Find the best feed composition for lactating cows and assume that throughout

- 50-60% forage

- 40-50 grain

scenarios - wastewater reuse technology, renewable energy,