### Model Window

- Where you build diagrams of your system.

- You’ll see levels (stocks), rates (flows), auxiliaries, and connectors.

- Acts as both a drawing board and a workspace.

### Sketch Tools Toolbar

- Found on the left side of the window.

- Tools include:

  - Level (Rectangle): Represents accumulations (e.g., population, money).

  - Rate (Pipe/Valve): Represents change over time (inflows/outflows).

  - Auxiliary (Circle): Represents variables that support calculations.

  - Connector (Arrow): Shows relationships between variables.

### Equation Editor

- Appears when you double-click a variable.

- Lets you define equations for each variable (e.g., Births = Population * Birth Rate).

- Built-in functions include IF THEN ELSE, SMOOTH, DELAY, etc.

### Control Panel (Simulation)

- Used to set up and run simulations.

- You can define:

  - Simulation time (start, stop, timestep).

  - Initial values for stocks.

  - Scenarios and sensitivity analysis.

### Graph/Output Window

- Shows simulation results as:

  - Graphs (line charts, bar charts).

  - Tables (numerical values for each timestep).

- Useful for comparing different scenarios.

## Sketch Tools and Icons

The toolbar is key to building models. Here’s a quick rundown:

|  | Name | Use |
| --- | --- | --- |
|  | Level (Stock) | Stores quantities (e.g., "Population"). |
|  | Rate (Flow) | Defines inflow/outflow (e.g., "Births"). |
|  | Auxiliary | Intermediate calculation variables (e.g., "Birth Rate"). |
|  | Connector Arrow | Links variables (e.g., Birth Rate → Births). |
|  | Text Tool | Add labels and documentation. |

## Defining Equations

Every variable needs an equation.

Stocks (Levels): Usually start with an initial value and update based on inflows/outflows.
 Example:

 Population = INTEG(Births - Deaths, 100)

- Here, 100 is the initial population.

Flows (Rates): Define the rate of change.
 Example:

 Births = Population * Birth Rate

Auxiliaries: Provide constants or formulas.
 Example:

 Birth Rate = 0.02

Running Simulations

- Click Simulate → Run a Simulation.

- Set:

  - Start Time (e.g., 0).

  - Stop Time (e.g., 100).

  - Time Step (e.g., 1).

- Run → Vensim generates data for all variables.

- Open the graph tool to plot results.

  - Compare multiple variables (e.g., Population vs Births).

## Analysis Tools

Vensim PLE includes several analysis features:

- Causes Tree: Shows which variables influence a chosen variable.

- Loops Tool: Highlights feedback loops.

- Sensitivity Runs (limited in PLE): Explore effects of changing parameters.

- Model Check: Validates equations for errors or missing definitions.

## Limitations of PLE

- Only one model can be open at a time.

- No advanced optimization tools (like in Vensim DSS).

- Limited sensitivity analysis.

- Cannot build subscripts (arrays of variables).

- Intended for learning, not commercial use.

## Main Ideas

Model Window: Where you draw and connect variables.

Sketch Tools: Build stocks, flows, auxiliaries, connectors.

Equation Editor: Define mathematical relationships.

Control Panel: Run simulations with chosen parameters.

Output Window: Graphs/tables for results and comparisons.

Sources:

- https://www.youtube.com/watch?v=uIcnEFQlBgw

- https://www.youtube.com/watch?v=30CxiyU0itY&list=PLrSp4pjHugDNRoL-BgV431OfOup2WbhGL&index=3

- https://www.youtube.com/watch?v=G58ESIKTctY&list=PLrSp4pjHugDNRoL-BgV431OfOup2WbhGL&index=1&pp=iAQB

- https://youtu.be/eM3Xzu8sr-8